PREFLIGHT

Medical, Biannual, IFR 	– Current

Annual, Transponder, ELT – Current

ARROW, VOR 30 day check – Current

Parking brake 		- Set

Mags			- Off

Elevator trim	           	- Neutral

Flight controls		- Free

Mixture			- Cutoff

Throttle			- Idle

Avionics master		- Off

Electrical switches		- Off

Master switch		- On

Flaps			- Check

Lights (beacon, nav, landing) - Checked

Fuel quantity		- Checked

Pitot heat			- Check as req

Master switch		- Off

FUSELAGE/EMPENNAGE

Baggage door   		- Secured

Belly, antennas 		- Checked

Stabilizers			- Checked

Elevator/Trim tab		- Checked

Tie down			- Removed

RIGHT WING

Fuel sump	 	 	- Drained

Landing gear/brakes	- Checked

Flaps/Aileron		- Checked

Wing tip, leading edge	- Checked

Tie down			- Removed

Fuel quantity/ Caps	- Check/Secure

NOSE

Oil quantity (6-8 qts)	- Checked

Exhaust			- Check for cracks

Cowling			- Secured

Prop/Nose wheel/Alt belt	- Checked

Air filter			- Clear

Static port			- Clear

LEFT WING

Fuel quantity/ Caps		- Check/Secure

Pitot tube 			- Checked

Tie down			- Removed

Stall horn			- Checked/Clear

Fuel vent			- Clear

Leading edge, wing tip	- Checked

Flaps/Aileron			- Checked

Landing gear/brakes		- Checked

Fuel sump 			- Drained

BEFORE ENGINE START

Checkout Plane		- Completed

Seatbelts			- On

Brief passengers		- Belts/Emergency

Organize cockpit (charts, etc)	- Complete

Fuel selector			- Both

Circuit breakers		- In

Electric switches		- Off

Doors/windows		- Secured

ENGINE START

Fuel Selector			- Both

Mixture			- Rich

Throttle			- 1/4” open

Carb heat			- Off

Prime				- as required

Master switch			- On

Beacon and Taxi (lights as needed)	- On

Brakes				- Set

CLEAR PROP & Start.

Idle				- 900-1000 rpm

Oil pressure/temp & ammeter - Checked

Flaps				- Up

Avionics master		- On

Transponder			- Standby/VFR

DG & clock			- Set

Mixture			- Lean for taxi

Record				- Time

ATIS/Radios 			- Set

Check brakes, DG, T&B, AI, compass during taxi.

RUN UP

Heading into the wind

Brakes				- Set

Flight controls			- Free & correct

Fuel Selector			- Both

Mixture			- Full Rich

Primer				- In and Locked

Throttle			- 1700 rpm

Mags	(125/50)		- Checked

Carb heat			- Checked

Oil Pressure			- Green

Oil Temp			- Green

Amperage			- Charging

Suction			- Normal(4.6-5.4)

Throttle			- Idle, then 1000 rpm

Mixture			- Lean for taxi

BEFORE TAKEOFF

Fuel Selector			- Both

Trim				- Set

Flaps				- As required

Mixture			- Rich

Carb heat			- Off

Altimeter			- Set

DG				- Set

Transponder			- Code set, ALT

Radios/Navigation		- Set

Doors				- Locked

Windows			- Closed

TAKE OFF

Vr			   	(63 MPH)

Vx			   	(68 MPH)

Vy				(90 MPH)

CLIMB

Enroute climb		   	   (80 MPH)

Flaps					- Up

Landing light				- Off

CRUISE

Throttle			- Set

Mixture			- As needed

BEFORE LANDING

Pattern 			- 2050rpm

Carb heat		           - On midfield/Descent

Altimeter			- Set

Mixture			- As required

Fuel selector		- Both

Primer			- In and Locked

Seat belts			- On

Landing Lights / strobes	- As required

90 mph downwind

80 mph base

70-75 mph final

65-70 mph over the numbers

CLEAR OF RUNWAY

Carb heat			- Off

Flaps			- Up

Trim 			- Neutral

Landing lights		- As required

Mixture			- Lean for taxi

ENGINE SHUTDOWN

Throttle			- 1000rpm

Transponder		- Standby/VFR

Avionics master		- Off

Electric switches		- Off

Mixture			- Cutoff

Mags			- Off

Master switch		- Off

Check-in Plane 		- Completed

CLOSE FLIGHT PLAN! De-bug

Position Report

Who you are?

Where are you?

What do you want to do?

Plane Requirements

VFR- TOMATOFLAMES

Tachometer

Oil Pressure

Manifold Pressure

Altimeter

Temperature Sensor (Liquid Cooled)

Oil Temperature (Air Cooled)

Fuel Gauge

Landing gear indicator

Airspeed indicator

Magnetic Compass

ELT

Seat Belts

Night VFR- FLAPS

Fuses or Circuit Breakers

Landing Light (if for hire)

Anti-collision lights

Position Lights

Source of Electricity

Cross Country Flights- RAWFAT

Runway Lengths

Alternates

Weather

Fuel Requirements

ATC Delays

Takeoff/Landing distances

Springfield Frequencies

Clearance 			121.7

ATIS		  	            127.65

Ground	 	            121.9

Tower		   	            121.3

Departure/Approach		126.15

Emergency Procedures- ABCDE

Airspeed

Best field

Checklist

Airpseed – 75 MPH

Carb Heat – ON

Fuel Selector – BOTH

Mixture – RICH

Ignition Switch –Both (start) Cycle Mags

Primer – IN and LOCKED

Declare – 121.5 or current ATC frequency mayday 	mayday, location, squawk 7700

Execute – Mixture Cutoff

Light Gun Signals

-Steady Green: on the ground, Cleared for Takeoff. In Flight, Cleared to Land.

-Flashing Green, on the Ground, Clear to taxi, in flight, return to land.

-Steady red, on the ground, stop. In flight, give way to other aircraft.

-Flashing red, taxi clear of runway, in flight, airport unsafe, do not land.

-Flashing white tells you return to your starting point and it only applies on the ground.                      -Alternating red and green means exercise extreme caution, both on the ground and in flight.

Airspeeds

Vne-    182 MPH

Vno- 	145 MPH

Va-      112 MPH* Changes with weight

Flaps- 	54-100 MPH

Vs- 	57 MPH

Vso-	49 MPH

Vr-	63 MPH

Vg- 	75 MPH

Squawks

VFR				1200

Hijack				7500

Lost Comms			7600

Emergency			7700