PREFLIGHT

Medical, Biannual, IFR 	– Current

Annual, Transponder, ELT 	– Current

ARROW, VOR 30 day check – Current

Parking brake 			- Set

Mags				- Off

Elevator/Rudder trim		- Neutral

Flaps				- Check

Flight controls			- Free

Mixture			- Cutoff

Throttle			- Idle

Avionics master		- Off

Electrical switches		- Off

Master switch			- On

Lights/Stall Horn           	- Checked

Fuel quantity			- Checked

Master switch			- Off

RIGHT WING

Flaps/Aileron			- Checked

Wing tip, leading edge	- Checked

Fuel quantity/ Caps		- Check/Secure

Tie down			- Removed

Fuel sump			- Drained

Landing gear/brakes		- Checked

NOSE

Oil quantity (6-8 qts)		- Checked

Cowling			- Secured

Prop/Nose wheel/Alt belt	- Checked

Fuel sump			- Drained

LEFT WING

Fuel sump			- Drained

Landing gear/brakes		- Checked

Tie down			- Removed

Pitot-static/drain		- Checked

Fuel quantity/ Caps		- Check/Secure

Stall warning tab		- Checked

Leading edge, wing tip	- Checked

Flaps/Aileron			- Checked

EMPENNAGE

Left/Right Stabilator Bolt	- Checked

Stabilator/Trim tab		- Checked

Tie down			- Removed

RIGHT FUSELAGE

External Power Door		- Closed

Baggage Door			- Closed/locked

BEFORE ENGINE START

Seatbelts			- On

Brief passengers		- Complete

Organize cockpit (charts, etc)	- Complete

Fuel selector			- Fullest tank

Circuit breakers		- In

Electric switches		- Off

Flaps				- Up

ENGINE START

Checkout Plane	            - Completed

Mixture			- Rich

Carb heat			- Off

Master switch			- On

Fuel pump			- On/Pressure

Prime				- as required

Beacon (lights as needed)	- On

Brakes				- Set

Throttle			- ½” open

CLEAR PROP & Start

Idle				- 1000 rpm

Oil pressure/temp & ammeter – Checked

Electric  Fuel pump		- Off

Mixture			- Lean for taxi

Avionics master		- On

Transponder			- Set/standby

DG & clock			- Set

Altimeter/Trim/Radios	- Set

Check brakes, DG, T&B, AI, compass during taxi.

RUN UP

Brakes				- Set

Flight controls			- Free & correct

Mixture			- Full Rich

Throttle			- 2000 rpm

Mags	(125/50)		- Checked

Carb heat			- Checked

Suction			- Normal (4.6-5.4)

Engine instruments		- Checked

Throttle			- 900-1000 rpm

Mixture			- Lean for taxi

BEFORE TAKEOFF

Fuel 				- Fullest tank

Trim				- Set

Flaps				- As required

Mixture			- Rich

Carb heat			- Off

Altimeter			- Set

DG				- Set

Transponder			- Squawk in, Alt

Electric Fuel pump		- On

Landing Light			- On

Autopilot			- Off

Door				- Locked

Window			- Closed

Strobes			- As needed

TAKE OFF/CLIMB

Vr	55 knots

Vx	64 knots

Vy	76 knots

Enroute climb			90 knots

Flaps				- Up

CRUISE

Electric Fuel Pump/Landing Light	- Off

Mixture			- Leaned

Navs/Radios/Charts 		- Set

BEFORE LANDING

Pattern				– 2000 rpm

Carb Heat		- On, midfield or Descent

Altimeter			- Set

Mixture			- As required

Fuel pump			- On

Autopilot			- Off

Fuel selector			- Fullest tank

Seat belts			- On

Landing Lights / Strobes	- On

Under 100 Knots downwind

80 Knots base

75 Knots final

66 Knots over the numbers

CLEAR OF RUNWAY

Flaps				- Up

Trim 				- Neutral

Fuel pump			- Off

Transponder			- Standby

Mixture			- Lean for taxi

Landing lights			- On

ENGINE SHUTDOWN

Throttle			- 1000rpm

Avionics master		- Off

Electric switches		- Off

Mixture			- Cutoff

Mags				- Off

Master switch			- Off

Record Tach/Hobbs time	- Completed

CLOSE FLIGHT PLAN! De-bug

Position Report

Who you are?

Where are you?

What do you want to do?

Plane Requirements

VFR- TOMATOFLAMES

Tachometer

Oil Pressure

Manifold Pressure

Altimeter

Temperature Sensor (Liquid Cooled)

Oil Temperature (Air Cooled)

Fuel Gauge

Landing gear indicator

Airspeed indicator

Magnetic Compass

ELT

Seat Belts

Night VFR- FLAPS

Fuses or Circuit Breakers

Landing Light (if for hire)

Anti-collision lights

Position Lights

Source of Electricity

IFR- GRABCARD

Generator

Radios

Attitude indicator

Ball

Clock

Adjustable Altimeter

Rate of turn indicator

Directional gyro

Cross Country Flights- RAWFAT

Runway Lengths

Alternates

Weather

Fuel Requirements

ATC Delays

Takeoff/Landing distances

Emergency Procedures- ABCDE

Airspeed

Best field

Checklist

Airpseed -- 76 Knots

Carb Heat – ON

Fuel Selector – BOTH

Mixture – RICH

Ignition Switch –Both (start)

Primer – IN and LOCKED

Declare/Squawk		121.5/7700

Execute

Squawks

Hijack				7500

Lost Comms			7600

Emergency			7700

Airspeeds

Vne – 154 Knots

Vno – 125 Knots

Va – 113 Knots 2550lbs; 89 Knots 1634lbs

Flaps – 49-102 Knots

Vs- 55 Knots

Vso – 49 Knots

DMMS- 78 Knots

Light Gun Signals

-Steady Green: on the ground, Cleared for Takeoff. In Flight, Cleared to Land.

-Flashing Green, On the Ground, clear to taxi, in flight, return to land.

-Steady red, on the ground, stop. In flight, give way to other aircraft.

-Flashing red, taxi clear of runway, in flight, airport unsafe, do not land.

-Flashing white tells you return to your starting point and it only applies on the ground.                      -Alternating red and green means exercise extreme caution, both on the ground and in flight.